### İNTERNET SAYFASI  https://srituhobby.com/how-to-make-a-rfid-door-lock-with-arduino/

### VİDEOSU https://www.youtube.com/watch?v=GOO84CGBPz8

### Rif kartı tarama kodu

/*RFID tag scan code

* https://srituhobby.com

*/

#include <LiquidCrystal_I2C.h>

#include <SPI.h>

#include <MFRC522.h>

#define RST_PIN 9

#define SS_PIN  10

byte readCard[4];

byte a = 0;

LiquidCrystal_I2C lcd(0x27, 16, 2);

MFRC522 mfrc522(SS_PIN, RST_PIN);

void setup() {

Serial.begin(9600);

lcd.init();

lcd.backlight();

while (!Serial);

SPI.begin();

mfrc522.PCD_Init();

delay(4);

mfrc522.PCD_DumpVersionToSerial();

lcd.setCursor(2, 0);

lcd.print("Put your card");

}

void loop() {

if ( ! mfrc522.PICC_IsNewCardPresent()) {

return 0;

}

if ( ! mfrc522.PICC_ReadCardSerial()) {

return 0;

}

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Scanned UID");

a = 0;

Serial.println(F("Scanned PICC's UID:"));

for ( uint8_t i = 0; i < 4; i++) {  //

readCard[i] = mfrc522.uid.uidByte[i];

Serial.print(readCard[i], HEX);

Serial.print(" ");

lcd.setCursor(a, 1);

lcd.print(readCard[i], HEX);

lcd.print(" ");

delay(500);

a += 3;

}

Serial.println("");

mfrc522.PICC_HaltA();

return 1;

}

### Programın tamamı

/*Door lock system code

* https://srituhobby.com

*/

#include <Servo.h>

#include <LiquidCrystal_I2C.h>

#include <SPI.h>

#include <MFRC522.h>

#define SS_PIN 10

#define RST_PIN 9

String UID = "A3 80 34 16";

byte lock = 0;

Servo servo;

LiquidCrystal_I2C lcd(0x27, 16, 2);

MFRC522 rfid(SS_PIN, RST_PIN);

void setup() {

Serial.begin(9600);

servo.write(70);

lcd.init();

lcd.backlight();

servo.attach(3);

SPI.begin();

rfid.PCD_Init();

}

void loop() {

lcd.setCursor(4, 0);

lcd.print("Welcome!");

lcd.setCursor(1, 1);

lcd.print("Put your card");

if ( ! rfid.PICC_IsNewCardPresent())

return;

if ( ! rfid.PICC_ReadCardSerial())

return;

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Scanning");

Serial.print("NUID tag is :");

String ID = "";

for (byte i = 0; i < rfid.uid.size; i++) {

lcd.print(".");

ID.concat(String(rfid.uid.uidByte[i] < 0x10 ? " 0" : " "));

ID.concat(String(rfid.uid.uidByte[i], HEX));

delay(300);

}

ID.toUpperCase();

if (ID.substring(1) == UID && lock == 0 ) {

servo.write(70);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Door is locked");

delay(1500);

lcd.clear();

lock = 1;

} else if (ID.substring(1) == UID && lock == 1 ) {

servo.write(160);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Door is open");

delay(1500);

lcd.clear();

lock = 0;

} else {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Wrong card!");

delay(1500);

lcd.clear();

}

}